# Digital PI Control Implementation of a Second-Order Analog Plant Using STM32F411RE

PROJECT REPORT

Shaikh Mohammad Umar

BACHELOR OF ENGINEERING

in

ELECTRONICS & COMMUNICATION ENGINEERING

GOVERNMENT ENGINEERING COLLEGE, BHARUCH

# Objective, Project Overview, and Introduction

### 1.1 How the Idea Originated

The idea for this project began with a fundamental question: How does a theoretical control system actually behave when implemented in real hardware?

In textbooks, transfer functions, pole locations, damping ratios, and settling times look clean and predictable. However, real systems are influenced by non-ideal components, implementation limits, sampling effects, and hardware constraints. The motivation behind this project was to bridge that gap — to move from mathematical modeling to a physically realized closed-loop control system using an embedded platform.

Instead of controlling a conventional mechanical plant such as a DC motor, the goal evolved into something more insightful: to realize a second-order dynamic system electrically, using analog circuit design, and then control it digitally using an STM32F411RE microcontroller.

This approach allows full visibility into:

- System dynamics

- Stability behavior

- Damping manipulation

- Closed-loop pole placement

- Digital controller implementation

- Real-time embedded execution

The core objective was not just to make a system work, but to understand why it behaves the way it does.

# Project Objective

The primary objective of this project is:

To design, analyze, and implement a closed-loop control system using the STM32F411RE microcontroller, where the physical plant is realized through an analog active filter circuit that emulates a second-order dynamic system.

The project aims to:

- Achieve zero steady-state error

- Limit overshoot to 15%

- Ensure settling time within 1 second (±2% criterion)

- Analyze damping behavior and pole dominance

- Implement a real-time digital PI controller

- Integrate analog and digital domains into a single working control architecture

This is a complete end-to-end control system realization — from modelling to hardware to firmware.

# 1.3 System Concept and Architecture

The system consists of two major parts:

1. Analog Plant (Physical System)

Instead of using a motor or mechanical setup, the plant is realized using a Sallen-Key active low-pass filter, carefully designed to match a desired second-order transfer function.

2. Digital Controller (STM32F411RE)

The controller:

- Reads plant output via ADC

- Computes error

- Implements a discrete-time PI algorithm

- Outputs PWM signal to drive the plant

- Executes inside a timer interrupt at fixed sampling time

This structure mirrors an industrial digital control system.

# 1.4 Understanding the Dynamics First

Before selecting components, a full system analysis was performed.

The required closed-loop performance specifications were:

- Zero steady-state error

- 15% overshoot

- Settling time ≈ 1 second

- One dominant second-order pair

- Additional real pole at least 10× farther left (−40)

This resulted in the desired characteristic equation:

(s+40)(s^2+8s+60)

From:

s^2+8s+60s

We obtain:

- ω_n^2= 60

- 2ζωn=8

This defines the damping and natural frequency of the dominant dynamics.

A critical insight during analysis was:

Even though the third pole is at −40 and not at −∞, it still influences system response.
It contributes slower tail dynamics, causing settling time slightly longer than ideal second-order predictions.

This confirmed that the system behaves like a heavily damped second-order system with a far but finite third pole.

# 1.5 Why Not Use an RLC Circuit?

Initially, the plant was explored using a passive RLC circuit to directly model a second-order system.

Three output possibilities were evaluated:

- Output across capacitor → Low-pass behavior

- Output across resistor → Band-pass behavior

- Output across inductor → High-pass behavior

However:

- Inductor output introduced zero at origin → high-pass

- Resistor output created band-pass response

- Passive implementation had gain limitations

- Matching exact coefficients became impractical

- Physical inductors introduce parasitic and computational inaccuracies

Conclusion:

Passive RLC realization lacked flexibility and precision.

This led to a strategic pivot.

# 1.6 Transition to Active Sallen-Key Realization

To gain control over damping and gain, the plant was redesigned using a Sallen-Key low-pass filter.

Transfer function:

Ks2R1R2C1C2+ sR1C1+ R2C1+ R1C21 - K+ 1

By equating this with the desired plant equation:

- R1R2C1C2=0.2

- R1C1+R2C2+R1C2(1-K)=2

Standard industry capacitor values were selected:

C1=C2=10μF

Solving the equations yielded:

- R1​≈7.65kΩ

- R2​≈261kΩ

Using E12/E24 standard values:

- R1​=7.5kΩ

- R2​=270kΩ

This provided a practical and manufacturable design.

# 1.7 Inner Rated Feedback Loop

The system includes:

- Main error feedback loop

- Inner derivative (rate) feedback loop

The rated feedback modifies the damping term.

From analysis, the coefficient of sss in the denominator became:

2+10Tds​

By tuning:

Tds≈0.08s to 0.2s

Damping can be adjusted physically using a potentiometer in the feedback path.

This gives real-time tuning capability of system damping.

# 1.8 Controller Strategy

A PI controller was selected because:

- Zero steady-state error is required

- Integral action ensures DC accuracy

- Derivative action is partially handled via rated feedback

Initial tuning parameters:

- Kp=7.5

- Ti = 0.16

- Td = 1.7 (evaluated but PI selected for implementation simplicity)

The controller is implemented digitally inside a timer interrupt, ensuring fixed sampling time.